FDOTConnect (ORD) and FDOTSS10/SS4/SS2/V7 (V8i) Right of Way Maps

Existing Right of Way Maps

Converting existing Right of Way (ROW) Maps into ORD can be done with various levels of results being the end product.  Because all right of way map projects are not assembled the same way for plotting, a decision must be made on what the desired end result of the conversion will be.  In FDOT many right of way map DGN sheet files are stand-alone sheets with no references.  These projects are simple to convert.  If the sheets are tied by reference to the Master CADD file, trying to duplicate the structure of the original right of way map project could be complex depending on the project references and DGN models being used.  However, by using the merge to master functionality of reference files, the complex Right of Way Map project can be easily converted into stand-alone sheets that can be revised when needed.

Stand-Alone Right of Way Map Sheets

Because it is not necessary to duplicate into ORD a complex mapping structure to revise a right of way map sheet, it is recommended that all projects prior to ORD be converted into an ORD right of way map project using the “Stand-Alone” sheet process.

Master CADD File Conversion

Note, that if a Master CADD file is in SS10 or prior versions of MicroStation (V8i) and is intended to be used to create new right of way map sheets in ORD, the Master CADD File should be converted.  The conversion process for a Master CADD file is quite different from “Stand-Alone” sheets.  Many tools in FDOTConnect are predicated on creating sheet models to cut sheets.  Therefore, a Master CADD file would need to be converted into a specific format for use in FDOTConnect.

New Master CADD File

Because use of FDOTConnect tools will be needed in a new Right of Way Mapping Project a new ORD Master CADD file must be created or converted as outlined below in the ORD Master CADD file section.

ORD and SS10 Scaling differences

Due to a scaling difference between previous FDOT V8i DGN files and FDOT Connect, all cells, custom line styles, and text had to be scaled down for FDOTConnect and ORD to use correctly.  This scaling difference is why using sheet models in ORD is so important.  And sheet models are important because the tools CADD Support is creating to assist with right of way mapping projects, including the Print Set Manager, are intended to work with Sheet Models and will not work in Design or Drawing Models.

The side effect of this scaling difference is, custom line styles and annotation text may be observed to be much too small, or much too large, perhaps covering the screen, when opened or referenced into an FDOTConnect ORD file.  When the file is much older, for example SS2, the names and looks of the embedded custom line styles are different than current line styles.  In this case you may observe that all custom line styles appear to be zero (0) line style which is a solid line.  The effect observed will depend on DGN settings in the original file, the age of the DGN and what version of MicroStation was originally used to create the sheet.  In all cases the fix is the same.

Another side effect of this difference in scaling is that annotation scale text can also appear to be much too large.  The fix to this is also simple in that the text property “Is Annotation” can manually be set to “False”.

Below are some steps and guidelines on converting SS10 and prior (V8i) right of way map files to FDOTConnect ORD format.  Note that these processes do not include converting or importing GPK data.

Converting FDOTSS10/SS4/SS2/V7 Sheets To FDOTConnect (ORD) Stand-Alone Sheets

First Step – Identify the existing Right of Way Project DGN files and Create the Same Files in ORD.

- Identify all DGN files to be converted.  In most cases the right of way project will consist of a Master CADD file and a series of DGN Design files that were used to create each sheet of the project.  Each DGN must be converted individually.

- An FDOTConnect “Workset” of the project must be created when opening FDOTConnect10.10 or later.  This Workset is typically named the same as the WPI number of the Right of Way Mapping project as stored in VAULT.  Use the FDOT Create Workset-DGN document to create a new FDOTConnect10.10 Workset and new blank ORD versions of the individual DGN files to be converted.  This includes the Master CADD file.  This process will NOT involve the use of FDOTSS10.

Second Step – Bring the SS10 DGN sheet file elements into the Corresponding ORD DGN sheet files.

- Working inside of the appropriate Workset, open the FDOTConnect ORD sheet DGN with the same name as the file you will be converting.

- Set the drawing scale of the ORD file to match the sheet that will be referenced and converted.  Once the scale is set, Save Settings and proceed to number 8).  If you do not know the scale of the sheet being referenced, then reference the file at the current default scale for now.

- Reference the V8i right of way map sheet DGN file you wish to convert.  Make sure Live Nesting settings and Nesting Depth accommodate all the references and the elements that must be visualized to plot the sheet.  Newer Right of Way Map sheets usually will have text and arrow heads obviously out of scale.  Sometimes they don’t immediately show but the problem is there.

- Determine the plotting scale of the referenced right of way map sheet file.  Zooming in on the title block or north arrow should give you the scale.  Note that selecting a custom line style such as a leader with an arrowhead or right of way line (RWLine level) will also show the plotting scale in the properties dialogue box under Line Style Parameters.  It is possible that some of the custom line styles have a different scale for plotting purposes, so it is prudent to check enough elements to determine the actual plotting scale.  Also, selecting known text heights will indicate the original plotting scale.

- Once the scale is determined, detach the reference file, and go back to number 4) above.

- Reference the V8i right of way map sheet DGN file you wish to convert.  Make sure Live Nesting settings and Nesting Depth accommodate all the references and the elements that must be visualized to plot the sheet.  Newer Right of Way Map sheets will have text and custom line style arrowheads obviously out of scale.  Sometimes they don’t immediately show but the problem is there.  Closing ORD and reopening the DGN file will usually show the scaling issues.

- In the Reference File dialogue box, select the referenced DGN that you wish to convert.  Click on the highlighted “Use Active Annotation Scale” button at the bottom of the reference File dialogue box.  This will un-highlight the button and possibly resolve the text and/or the line style scale difference issues when merging into master file.

- In the Reference File dialogue box, select to highlight the referenced DGN that you wish to convert.  Select Tools > Merge Into Master.  Then left click inside of the view with the referenced file and then select “OK” if the warning dialogue box pops up.

Third Step – Adjust Custom Line Styles and Annotation Scale Text

- Once the SS10 DGN elements have merged into the ORD version of the file, the SS10 file will automatically detach as a reference.  You now have inside a single ORD version of the file all SS10 elements.  This is where you may see the scaling issues.

- At this point it is recommended that you close ORD completely and reopen the file.

When merging into master, ORD will try to emulate the scale of the original reference with varying degrees of success depending on the original V8i file.  ORD will try to scale based on What You See Is What You Get (WYSIWYG).  Because of edits and revisions to FDOT line styles and scales over the years and specific file settings when drawn, it can be very close or way off.

Depending on the age of the file and file settings, you may see custom line styles such as arrow heads way too large, or way too small.  If you see line style scaling issues, you will need to set all custom line styles to a scale of 1.0 to get them back to their intended appearance.  Note that it is common practice to scale up or down line styles to accommodate short lines.  The functionality to modify individual line style scales is still available in ORD.

Text is a different issue.  The original file may not have used annotation scale however, there are instances of embedded annotation scale text in the sheet cells.

- Select all Elements in the ORD file and observe at the top of the properties dialogue box, that all elements selected are shown, with details in the lower section.

- In the element properties, find the Arcs, Line Strings, Lines and Text groups.  Each group will need to be modified; however, you will need to change each group one at a time.

- Select the Arcs and find the property, Line Style Parameters and expand to show the current scale.  Change the scale of all Arcs to 1.0.  Repeat this process with Line Strings and Lines.  If the text is out of scale, select the Text and find the property, Is Annotation and set it to False.

- If necessary, modify the scale of any line styles that may appear too big at 1.0 for the short distance that is visualized.  This is a manual process to change the individual line style scale.  For example, 0.5 to half the size.

Converting FDOTSS10/SS4/SS2/V7 Master CADD File To FDOTConnect (ORD)