| Inventory ID | District | County | State Road Number |
| --- | --- | --- | --- |
| Section | Sub-Section | Roadway ID | Mile Post |
| End Treatment Type: |  |  |  |

| Compliant |  |  | Specifications |  |  |
| --- | --- | --- | --- | --- | --- |
| Yes | No | N/A |  |  |  |
| ☐ | ☐ |  | - Verified physical End Treatment matches manufacturer’s manual and/or drawings. |  |  |
| ☐ | ☐ | ☐ | - Confirm guardrail was installed in accordance with Standard Plans Index 536-001, or previous applicable version. |  |  |
| ☐ | ☐ | ☐ | - No weeds, trees, trash, or obstacles are within striking distance of terminal or immediate area. |  |  |
| ☐ | ☐ | ☐ | - Impact head/face plate has correct measurements of retroreflective adhesive sheeting based on manual/drawings. |  |  |
| ☐ | ☐ | ☐ | - Wooden posts or blocks are not rotten, deteriorated, or leaning beyond design specifications. |  |  |
| ☐ | ☐ | ☐ | - Steel posts are not rusted, deteriorated, or bent/leaning beyond Standard Specifications Section 536. |  |  |
| ☐ | ☐ | ☐ | - All offset blocks are properly aligned. |  |  |
| ☐ | ☐ | ☐ | - Entire guardrail terminal is free from damage. |  |  |
| ☐ | ☐ | ☐ | - Soil, clay, or concrete is level with road/on-ramp/right-of-way and not washing away. |  |  |
| ☐ | ☐ | ☐ | - All wooden/steel posts are accounted for based off Standard Specifications Section 536. |  |  |
| ☐ | ☐ | ☐ | - Distance between each post complies with drawings. |  |  |
| ☐ | ☐ | ☐ | - Confirm height of guardrail is 27” or 31”. Enter measured height: _______ |  |  |
| ☐ | ☐ | ☐ | - All bolts, washers, nuts and bearing plates are accounted for based off design specifications and are properly installed in accordance with Standard Specifications Section 536. |  |  |
| ☐ | ☐ | ☐ | - Cable anchor is not bent, rusted, or broken and is attached tautly in appropriate locations based off Standard Specifications Section 536. |  |  |
| ☐ | ☐ | ☐ | - Guardrail sections are lapped in a downstream end facing manner. |  |  |
| ☐ | ☐ | ☐ | - Ground strut is above ground, connected properly, not rusted, and visible without obstruction of weeds, grass, soil, or trash. |  |  |
| ☐ | ☐ | ☐ | - Entire guardrail system is appropriately distanced from roadway based on speed specifications. |  |  |
| Notes: |  |  |  |  |  |
| Inspector Name: |  |  |  | Signature: | Date: |