VIEW → LIBRARY → BRIDGE EXPLORER → Import FDOT library

Get Library.brlx

- select “Library”

- click the “>>” button

- click the “Import” button

BTW

Factors, LRFR – Florida load factors and tests per BLRM.

LRFR DF Applicability Ranges – expanding the range of applicability for approximate distribution factor equations per SDG.

Prestress Shapes, I Beams, Wide Top Flange – adding Florida FIB; start with 36” and add height for taller FIBs.

Get FDOT settings

BRIDGE → BRIDGE EXPLORER → import the FDOT analysis templates file

Get Settings.brsx (select “System Data”  “>>”  and “Import”)

BTW

Analysis Event Template – several Florida-specific setups for LFR/LRFR analysis.

System Defaults – tells BrR to use FDOT_AnalysisPreferences (General tab)and FDOT LRFR factors (Specifications tab)

Apply FDOT settings

With the Library closed, VIEW → Configuration → System Defaults

(1) Default analysis preferences = FDOT

(2) For Superstructure Analysis, use FDOT range of applicability

(3) For all AASHTO LRFR, Factors = FDOT.

Select Columns

Bridge Explorer -> VIEW -> Select Columns (optional, to choose at will)

Notes on BrR settings. BrR is a real wander, for settings and tuning. Know these:

- EXPLORER WINDOW → BRIDGE EXPLORER → Preferences. Mostly this says where things are. Point to a local folder on a fast hard drive. That is, instead of performing the analysis at MyDocuments, which could be housed at a slow OneDrive external data center, try a local location like C:\BrR.

- EXPLORER WINDOW → TOOLS → General Preferences. “Open template” and “Edit preferences” to examine and tune settings. This section is very powerful, as it selects how BrR will assess the structure. For example, selecting iterative MCFT shear for prestressed beams and moment redistribution for continuous slabs can meaningfully improve accuracy. When a particular template is selected as a default (in View → Configuration), new bridges will be assessed with that template. However, an imported bridge from a friend will use the settings that the author applied; to change this, select that bridge, then TOOLS → General Preferences → Open Template → select and open → apply → yes.

- EXPLORER WINDOW → VIEW → LIBRARY

  - Shapes and things (beams, rails, bolts, concrete, timber, etc.)

  - LRFD DF applicability – acceptable ranges of applicability for the approximate distribution equations

  - Factors, especially FDOT-LRFR…

  - Vehicles

- EXPLORER WINDOW → VIEW → Configuration, especially:

  - System Defaults → General → Default preference setting = FDOT…

  - System Defaults → Superstructure… → LRFD DF applicability = FDOT

  - System Defaults → Specifications → Factors = FDOT… for all LRFR

- BRIDGE WINDOW → DESIGN/RATE → ANALYSIS SETTINGS → Advanced. Define permit conditions and load factors. Then save it as a template. Also in this area, notice “Apply preference setting,” and the “Advanced” button.

- BRIDGE WINDOW → SUPERSTRUCTURE DEFINITIONS → Double-Click on a superstructure unit name → “Analysis” tab

  - Longitudinal and transverse loading increment

  - Number of shell elements

- BRIDGE WINDOW → SUPERSTRUCTURE DEFINITIONS → Unit → MEMBER ALTERNATIVES → Double-Click on an alternative name → “Control Options” tab.

Notes, LRFR Factors. On 01-30-2025, the BrR 7.5.1 factors were revised – duplicating the most recent BrR resistance factors and applying several Florida-specific changes – mainly load factors from the BLRM-Summary Form at https://www.fdot.gov/maintenance/LoadRating.shtm; neglect Service I, neglect Fatigue, say all Legal and Permit load factors are 1.35, and say γDW=1.25 per field measured. BrR’s default capacity resistance factors were kept, as well as γrefined.loss.Service.III=1.00 (Florida prefers approximate losses at γapproximate.loss.Service.III=0.80). Also, resistance factors for prestressed tension-controlled post-tension concrete was input as 0.90 for both normal and lightweight concrete per LRFD 5.5.4.2—Resistance Factors.

Notes, DF Applicability (follow FDOT SDG 2.9)

Type “c” U-Beam 	18 < d < 72 	moment-interior, shear-interior, shear-skew

Type “c” U-Beam 	20 < L < 170	moment-interior, shear-interior, shear-skew

Type “f” FSB	26 < b < 63	moment-interior, shear-interior, shear-skew (SDG and add 3” more to allow room for gaps)

Type “f” FSB 	12 < d < 60	shear-skew

Type “f” FSB 	4 ≤ Nb ≤ 20	moment-interior, shear-interior, shear-skew

Type “f” FSB 	5,700 < I < 610E3	shear-interior

Type “k” I-Beam	10E3 < Kg < 8.5E6	moment-interior

Bridge Description & Custom Fields

Submit BrR input files to the FDOT with:

- Description tab - Bridge ID = NBI Structure ID.

- Description tab – Name. Structure type and span lengths.

- Description tab - Description. Analysis notes.

- Custom agency fields tab - Standard Index number(s) for the superstructure, or NA.

- Custom agency fields tab – Production. Say “no.” Later, when the Department accepts the rating as correct, and the bridge is open to traffic, this field will be recoded to “yes.”

BTW other BrR description fields are redundant, as they can be associated with Bridge Management System (BMS) data by Structure ID.

Shear

For shear, follow the FDOT Bridge Load Rating Manual (BMRM) at 6A.5.8, but use the Ultimate Demand Based Capacity (UDBC) for all rating Levels and all vehicles. Commentary: The BLRM limits the requirement to the FL120 where RFFL120 <  0.90 because it was a hardship to apply in other software. In BrR, the UDBC option is automated.

Culverts

- The Department recommends end conditions with 50 pci spring supports and sidesway support. The assessing engineer may use other support conditions, provided the narrative explains why (site-specific soil conditions, using fluid supports to match Iowa CulvertCalc check program, etc.). Commentary: Edge cases notwithstanding, springs are similar to fluid supports (see internal November 2025 files at “CBC_FLUIDvsSPRINGS”, and Improving_Culvert_Load_Rating_Process_and_Software from Texas at “…research showed this modeling approach increased the work of load rating but did not improve the accuracy of the results“). The guidance is provided for consistency.

- The Department does not recommend using the “Ignore effects from negligible LL” toggle (MBE 6A.10.10.3a). Instead, compute the load rating regardless. Commentary: Even when γLL < 10%(γDL+γLL), the Department still requires a load rating.

Culvert end conditions.

Culvert control options.