FDOT ICE Training

SR 710 at Northlake Boulevard – Case Study Data

- For Stage 1 – CAP-X Analysis

  - Existing Year – 2019

  - Truck Percentage

    - SR 710 – NB: 13.9%, SB: 14.8%

    - Northlake Boulevard – EB: 4.0%, WB: 9.8%

  - FDOT Context Classification – C3R Suburban Residential

  - See Page 3 for turning movement counts input

- For Stage 1 – SPICE Analysis

  - Opening Year – 2020

    - SR 710 AADT – 22,400

    - Northlake Boulevard AADT – 33,400

  - Design Year – 2040

    - SR 710 AADT – 28,700

    - Northlake Boulevard AADT – 38,800

  - Speed Limit

    - SR 710 – 55 MPH

    - Northlake Boulevard – 55 MPH

- For Stage 2 – SPICE

  - At-Grade Part C CMF Inputs

    - Lighting Present – Yes

    - # Approaches with Permissive LT Signal Phasing – 0

    - # Approaches with Perm./Prot. LT Signal Phasing – 0

    - # Approaches with Protected LT Signal Phasing – 3

    - # Approaches with RTOR Prohibited – 0

    - Red Light Cameras Present – No

    - Ped Volume – Low (50)

    - Max Lanes Crossed by Ped – 8

    - # Bus Stops within 1,000’ – 0

    - Schools within 1,000’ of intersection – No

    - # of Alcohol Establishments within 1,000’ – 0

  - Historical crash data is provided on Page 2

- For Stage 2 – ICE Tool

  - AM Peak Hour – 7:00 to 8:00 AM

  - PM Peak Hour – 5:00 to 6:00 PM

  - Facility Type – Urban Principal Arterial

  - Design and Construction Costs

    - Traffic Signal – $0

    - Partial DLT – $3,100,000

    - Quadrant Roadway – $1,810,000

  - ROW Costs

    - Traffic Signal – $0

    - Partial DLT – $1,700,000

    - Quadrant Roadway – $0

  - Obtain Safety information input from Stage 2 SPICE Tool

    - Crash Prediction Summary table in the Results tab

  - Delay Information

    - Traffic Signal –

      - Opening Year AM – 50.1 seconds

      - Opening Year PM – 89.5 seconds

      - Design Year AM – 190.3 seconds

      - Design Year PM – 234.2 seconds

    - Quadrant Roadway –

      - Opening Year AM – 41.6 seconds

      - Opening Year PM – 70.9 seconds

      - Design Year AM – 130.4 seconds

      - Design Year PM – 269.4 seconds

    - Partial DLT – information already provided in spreadsheet

Note: Orange cells represent truck %

US 41 at SR 44 – Case Study Data

- For Stage 1 – CAP-X Analysis

  - Existing Year – 2018

  - Truck Percentage

    - US 41 – NB/SB: 10.0%

    - SR 44 – EB: 6.8%, WB: 4.5%

  - FDOT Context Classification – C3C Suburban Commercial

  - See Page 7 for turning movement count inputs

- For Stage 1 – SPICE Analysis

  - Opening Year – 2020

    - US 41 AADT – 30,300

    - SR 44 AADT – 15,900

  - Design Year – 2040

    - US 41 AADT – 37,400

    - SR 44 AADT – 18,400

  - US 41 Speed Limit – 45 MPH

  - SR 44 Speed Limit – 45 MPH

- For Stage 2 – SPICE

  - At-Grade Part C CMF Inputs

    - Lighting Present – Yes

    - # Approaches with Permissive LT Signal Phasing – 0

    - # Approaches with Perm./Prot. LT Signal Phasing:

      - Traffic Signal – 3

      - Traffic Signal (Alt.) – 2

    - # of Approaches Protected LT Signal Phasing:

      - Traffic Signal – 1

      - Traffic Signal (Alt.) – 2

    - # Approaches with RTOR Prohibited – 0

    - Red Light Cameras Present – No

    - Ped Volume – Low (50)

    - Max Lanes Crossed by Ped – 6

    - # Bus Stops within 1,000’ – 0

    - Schools within 1,000’ of intersection – No

    - # of Alcohol Establishments within 1,000’ – 4

  - Roundabout

    - Leg 1

      - Opening Year AADT – 15,150

      - Leg has Right-Turn Bypass – Yes

      - Entering Width (ft) – 30

      - # of Entering Lanes – 2

      - # of Circulating Lanes – 1

    - Leg 2

      - Leg has Right-Turn Bypass – No

      - Entering Width (ft) – 30

      - # of Entering Lanes – 2

      - # of Circulating Lanes – 2

    - Leg 3

      - Opening Year AADT – 7,950

      - Leg has Right-Turn Bypass – No

      - Entering Width (ft) – 30

      - # of Entering Lanes – 2

      - # of Circulating Lanes – 2

    - Leg 4

      - Leg has Right-Turn Bypass – Yes

      - Entering Width (ft) – 30

      - # of Entering Lanes – 2

      - # of Circulating Lanes – 2

  - Restricted Crossing U-Turn

    - # U-Turns – 2

    - # of Major Roadway Lanes – 2

    - # of Minor Roadway Lanes – 2

    - Total Offset Distance (ft) – 1,250

    - Number of Driveways – 8

    - Total Deceleration Lane Length (ft) – 400

    - Number of Left-Turn Lanes From Major Road – 2+

    - Major Road Speed Limit (mph) – <=50

    - Total Median Width (ft) – 50

  - Historical crash data is provided on Page 6

- For Stage 2 – ICE Tool

  - AM Peak Hour – 7:00 to 8:00 AM

  - PM Peak Hour – 4:00 to 5:00 PM

  - Facility Type – Urban Principal Arterial

  - Design and Construction Costs

    - Traffic Signal - $0

    - Modified Signal - $790,000

    - Roundabout - $2,470,000

    - Signalized RCUT - $2,360,000

    - Quadrant Roadway - $1,500,000

  - ROW Costs

    - Traffic Signal - $0

    - Modified Signal - $0

    - Roundabout - $725,000

    - Signalized RCUT - $100,000

    - Quadrant Roadway - $2,000,000

  - Obtain safety input information from Stage 2 SPICE Tool

    - Crash Prediction Summary table in the Results tab

  - Delay Information

    - Traffic Signal -

      - Opening Year AM – 25.3 seconds

      - Opening Year PM – 29.3 seconds

      - Design Year AM – 30.8 seconds

      - Design Year PM – 49.2 seconds

    - Traffic Signal (Alt) –

      - Opening Year AM – 21.8 seconds

      - Opening Year PM – 26.6 seconds

      - Design Year AM – 26.2 seconds

      - Design Year PM – 46.6 seconds

    - Roundabout –

      - Opening Year AM – 8.3 seconds

      - Opening Year PM – 11.8 seconds

      - Design Year AM – 13.3 seconds

      - Design Year PM – 21.4 seconds

    - Quadrant Roadway –

      - Opening Year AM – 42.7 seconds

      - Opening Year PM – 40.7 seconds

      - Design Year AM – 58.3 seconds

      - Design Year PM – 60.0 seconds

    - Signalized RCUT – information already provided in spreadsheet

Note: Orange cells represent truck %