|  | FDOT Traffic Engineering Research Laboratory (TERL) NEMA TS4 2.2.3-2.2.6, 3.1.1 & 5.4 Checklist | By signing this form, the applicant declares that he/she has read and understands the provisions of the NEMA TS4 Standard. The requirements listed on this checklist are derived from Sections 2.2.3-2.2.6, 3.1.1, and 5.4 and are the basis for determining a product’s compliance to these sections of the standard. |
| --- | --- | --- |

| Date: | Click here to enter a date. | Applicant’s Name (print): |  |
| --- | --- | --- | --- |
| Manufacturer: |  |  |  |
| Item, Model No.: |  | Signature: |  |

|  |  | ** Greyed out rows in table below are for TERL use only ** |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID No | Section | Requirement | Item Comply? (Yes/No/NA) | Comments (Applicant must provide information as indicated) | TERL Evaluation Method |
| The following criteria are for the Temperature and Voltage Tests for Sign Beacons, Highlighted Signs, Electronic Display Signs (EDS), Dynamic Message Signs (DMS), Portable Arrow Boards, Portable Changeable Message Signs (PCMS), Portable Regulatory Signs, Portable Radar Speed Display Units, and Truck Mounted Changeable Message Signs. |  |  |  |  |  |
| The following criteria are for the Low-Temperature Low-Voltage Tests. |  |  |  |  |  |
| 1 | 2.2.3.1.2 | Test unit operates without malfunction with its input voltage adjusted to 97 VAC and the environmental chamber’s temperature is set to room temperature. |  | For devices powered by voltages other than 120 VAC, minimum voltages are: - 12 VDC: Min 11.2 VDC - 24 VDC: Min 22.4 VDC - 48 VDC: Min 44.8 VDC - 240 VAC: Min 214 VAC | Document Review |
|  |  |  |  |  | Init.: |
| 2 |  | Test unit cycles and the environmental chamber’s temperature is lowered to -34°C at a rate not exceeding 17°C per hour. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 3 |  | Test unit cycles for a minimum period of 5 hours and demonstrates the functions in Section 2.2.3. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 4 |  | Test unit’s power is removed for a minimum period of 5 hours and upon restoration of power the test unit undergoes its prescribed start-up sequence and continues cycling. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 5 |  | Test unit demonstrates the functions listed in Section 2.2.3. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for the Power Interruption Tests for DMS. |  |  |  |  |  |
| 6 | 2.2.6 | Test unit cycles in a continuous loop and its input voltage is removed for a period of 49 ms and test unit operates normally as though no power interruption has occurred upon restoration of power. Repeated three times. |  | Power interruption test is not required for devices powered by non-grid sources. | Document Review |
|  |  |  |  |  | Init.: |
| 7 |  | Test unit cycles in a continuous loop and its input voltage is removed for a period of 99 ms and test unit operates as stated in Table 2-5. Repeated three times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 8 |  | Test unit cycles in a continuous loop and its input voltage is removed for a period greater than or equal to 100 ms and test unit operates as stated in Table 2-5. Repeated three times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 9 | 2.2.6.1.1 | DMS power is cycled continuously for a period of 2 minutes where the power is off for 300 ± 50 ms and then on for 700 ± 50 ms. |  | The power to the DMS is interrupted through a non-wetted electromechanical contactor. | Document Review |
|  |  |  |  |  | Init.: |
| 10 |  | Test unit reverts to its start-up sequence (or continue normal operation) upon restoration of input power without operator intervention, and then continues to operate without malfunction. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 11 | 2.2.6.1.2 | Test unit cycles while slowly reducing the power line from nominal voltage to zero voltage and then back to minimum voltage, at a rate not exceeding 1 volt per second and upon restoration of input power (minimum voltage) test unit reverts to its start-up sequence or continues normal operation. Repeat five times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for the Low-Temperature High-Voltage Tests. |  |  |  |  |  |
| 12 | 2.2.3.2.2 | Test unit operates without malfunction with its input voltage adjusted to 135 VAC, the environmental chamber temperature remaining at -34°C and with humidity controls off, and cycles for one hour. |  | For devices powered by voltages other than 120 VAC, maximum voltages are: - 12 VDC: Max 16.5 VDC - 24 VDC: Max 28 VDC - 48 VDC: Max 55 VDC - 240 VAC: Max 264 VAC | Document Review |
|  |  |  |  |  | Init.: |
| 13 |  | Test unit demonstrates the functions listed in Section 2.2.3. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for the Power Interruption Tests for DMS. |  |  |  |  |  |
| 14 | 2.2.6 | Test unit cycles in a continuous loop and its input voltage is removed for a period of 49 ms and test unit operates normally as though no power interruption has occurred upon restoration of power. Repeated three times. |  | Power interruption test is not required for devices powered by non-grid sources. | Document Review |
|  |  |  |  |  | Init.: |
| 15 |  | Test unit cycles in a continuous loop and its input voltage is removed for a period of 99 ms and test unit operates as stated in Table 2-5. Repeated three times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 16 |  | Test unit cycles in a continuous loop and its input voltage is removed for a period greater than or equal to 100 ms and test unit operates as stated in Table 2-5. Repeated three times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 17 | 2.2.6.1.1 | DMSs power is cycled continuously for a period of 2 minutes where the power is off for 300 ± 50 ms and then on for 700 ± 50 ms. |  | The power to the DMS is interrupted through a non-wetted electromechanical contactor. | Document Review |
|  |  |  |  |  | Init.: |
| 18 |  | Test unit reverts to its start-up sequence (or continue normal operation) upon restoration of input power without operator intervention, and then continues to operate without malfunction. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 19 | 2.2.6.1.2 | Test unit cycles while slowly reducing the power line from nominal voltage to zero voltage and then back to minimum voltage, at a rate not exceeding 1 volt per second and upon restoration of input power (minimum voltage) test unit reverts to its start-up sequence or continues normal operation. Repeat five times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for the High-Temperature High-Voltage Tests with Humidity. |  |  |  |  |  |
| 20 | 2.2.3.3.2 | Test unit cycles without malfunction with an input voltage of 135 VAC and the environmental chamber’s temperature is increased to 74°C at a rate not exceeding 17°C per hour. |  | For devices powered by voltages other than 120 VAC, maximum voltages are: - 12 VDC: Max 16.5 VDC - 24 VDC: Max 28 VDC - 48 VDC: Max 55 VDC - 240 VAC: Max 264 VAC | Document Review |
|  |  |  |  |  | Init.: |
| 21 |  | Test unit cycles without malfunction with the humidity controls set to not exceed 95% relative humidity over the temperature range of 4.4°C to 43.3°C and when the temperature reaches 43°C, the humidity controls are readjusted to maintain constant absolute humidity. |  | Relative humidities for wet bulbs are shown in Table 2-4. | Document Review |
|  |  |  |  |  | Init.: |
| 22 |  | Test unit cycles for a minimum of 15 hours at 74°C and 18% relative humidity and then operates without malfunction. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 23 |  | Test unit demonstrates the functions listed in Section 2.2.3. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for the Power Interruption Tests for DMS. |  |  |  |  |  |
| 24 | 2.2.6 | Test unit cycles in a continuous loop and its input voltage is removed for a period of 49 ms and test unit operates normally as though no power interruption has occurred upon restoration of power. Repeated three times. |  | Power interruption test is not required for devices powered by non-grid sources. | Document Review |
|  |  |  |  |  | Init.: |
| 25 |  | Test unit cycles in a continuous loop and its input voltage is removed for a period of 99 ms and test unit operates as stated in Table 2-5. Repeated three times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 26 |  | Test unit cycles in a continuous loop and its input voltage is removed for a period greater than or equal to 100 ms and test unit operates as stated in Table 2-5. Repeated three times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 27 | 2.2.6.1.1 | DMS power is cycled continuously for a period of 2 minutes where the power is off for 300 ± 50 ms and then on for 700 ± 50 ms. |  | The power to the DMS is interrupted through a non-wetted electromechanical contactor. | Document Review |
|  |  |  |  |  | Init.: |
| 28 |  | Test unit reverts to its start-up sequence (or continue normal operation) upon restoration of input power without operator intervention, and then continues to operate without malfunction. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 29 | 2.2.6.1.2 | Test unit cycles while slowly reducing the power line from nominal voltage to zero voltage and then back to minimum voltage, at a rate not exceeding 1 volt per second and upon restoration of input power (minimum voltage) test unit reverts to its start-up sequence or continues normal operation. Repeat five times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for the High-Temperature Low-Voltage Tests with Humidity. |  |  |  |  |  |
| 30 | 2.2.3.4.2 | Test unit operates without malfunction with the input voltage adjusted to 97 VAC. |  | For devices powered by voltages other than 120 VAC, minimum voltages are: - 12 VDC: Min 11.2 VDC - 24 VDC: Min 22.4 VDC - 48 VDC: Min 44.8 VDC - 240 VAC: Min 214 VAC | Document Review |
|  |  |  |  |  | Init.: |
| 31 |  | Test unit demonstrates the functions listed in Section 2.2.3. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for the Power Interruption Tests for DMS. |  |  |  |  |  |
| 32 | 2.2.6 | Test unit cycles in a continuous loop and its input voltage is removed for a period of 49 ms and test unit operates normally as though no power interruption has occurred upon restoration of power. Repeated three times. |  | Power interruption test is not required for devices powered by non-grid sources. | Document Review |
|  |  |  |  |  | Init.: |
| 33 |  | Test unit cycles in a continuous loop and its input voltage is removed for a period of 99 ms and test unit operates as stated in Table 2-5. Repeated three times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 34 |  | Test unit cycles in a continuous loop and its input voltage is removed for a period greater than or equal to 100 ms and test unit operates as stated in Table 2-5. Repeated three times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 35 | 2.2.6.1.1 | DMSs power is cycled continuously for a period of 2 minutes where the power is off for 300 ± 50 ms and then on for 700 ± 50 ms. |  | The power to the DMS is interrupted through a non-wetted electromechanical contactor. | Document Review |
|  |  |  |  |  | Init.: |
| 36 |  | Test unit reverts to its start-up sequence (or continue normal operation) upon restoration of input power without operator intervention and then continues to operate without malfunction. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 37 | 2.2.6.1.2 | Test unit cycles while slowly reducing the power line from nominal voltage to zero voltage and then back to minimum voltage, at a rate not exceeding 1 volt per second and upon restoration of input power (minimum voltage) test unit reverts to its start-up sequence or continues normal operation. Repeat five times. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for Test Termination. |  |  |  |  |  |
| 38 | 2.2.3.5 | Test unit cycles through all functions and its input voltage is adjusted to 120 VAC. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 39 |  | Test unit cycles without malfunction and the environmental chamber’s temperature is set to return to room temperature at a rate not exceeding 17°C per hour. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 40 |  | Test unit operates without malfunction after stabilizing at room temperature for one hour. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for the Vibration Test for Sign Beacons, Highlighted Signs, EDS, DMS, Portable Arrow Boards, PCMS, Portable Regulatory Signs, Portable Radar Speed Display Units, and Truck Mounted Changeable Message Signs. |  |  |  |  |  |
| 41 | 2.2.4.1 | Test equipment meets the stated requirements. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 42 | 2.2.4.2 | Vibration table is set for a double amplitude displacement of 0.381 mm. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 43 |  | Vibration table cycles over a search range from 5 to 30 Hz and back within a period of 12.5 minutes. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 44 |  | Resonant sinusoidal frequency is determined in each of the three mutually perpendicular planes. |  | In the event of more than one resonant frequency in each plane, record the most severe resonance. If resonant frequencies appear equally severe, record each resonant frequency. If no resonant frequency occurs for a given plane within the prescribed range, record 30 Hz. | Document Review |
|  |  |  |  |  | Init.: |
| 45 | 2.2.4.3 | Test unit is vibrated in each plane at its resonant sinusoidal frequency for a period of 1 hour at an amplitude resulting in 0.5 g acceleration. |  | When more than one resonant frequency has been recorded in accordance with Section 2.2.4.2, equally divide the test period of 1 hour between the resonant frequencies. The total time of the endurance test is limited to 3 hours, 1 hour in each of the three mutually perpendicular planes. | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for the Shock Test for Sign Beacons, Highlighted Signs, EDS, DMS, Portable Arrow Boards, PCMS, Portable Regulatory Signs, Portable Radar Speed Display Units, and Truck Mounted Changeable Message Signs. |  |  |  |  |  |
| 46 | 2.2.5.1 | Test equipment meets the stated requirements. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 47 | 2.2.5.2 | Test unit is secured to the test table on one of its three mutually perpendicular planes and the test table is set to the calibrated shock level. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 48 |  | Test table is shocked at the calibrated level and repeated for each of the remaining two mutually perpendicular planes using the same calibrated shock level. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 49 |  | With the observations of the accelerometer for the three tests, complete the stated actions for the corresponding observed g’s. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for Weather-Tight Enclosure for DMS. |  |  |  |  |  |
| 50 | 3.1.1.1 | The DMS housing, including its face panels, meets the requirements of NEMA type 3R, as described in ANSI/NEMA 10250. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for Protection Against Solid Foreign Objects from ANSI/NEMA 10250. |  |  |  |  |  |
| 51 | ANSI/NEMA 10250 Section 5.2.1 | For a non-ventilated DMS with live parts less than 102 mm (4 in.) from an opening, insert a rod having a diameter of 3.2 mm (1/8 in.). |  | Passes if the probe can’t penetrate. | Document Review |
|  |  |  |  |  | Init.: |
| 52 |  | For a non-ventilated DMS with live parts 102 mm (4 in.) or more from an opening, insert a rod having a diameter of 12.7 mm (1/2 in.). |  | Passes if the probe can’t penetrate. | Document Review |
|  |  |  |  |  | Init.: |
| 53 | ANSI/NEMA 10250 Section 5.2.2 | For a ventilated DMS with live parts less than 102 mm (4 in.) from an opening, insert a rod having a diameter of 12.7 mm (1/2 in.). |  | Passes if the probe can’t penetrate. | Document Review |
|  |  |  |  |  | Init.: |
| 54 |  | For a ventilated DMS with live parts 102 mm (4 in.) or more from an opening, insert a rod having a diameter of 19 mm (3/4 in.). |  | Passes if the probe can’t penetrate. | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for Protection Against Ingress of Water from ANSI/NEMA 10250. |  |  |  |  |  |
| 55 | ANSI/NEMA 10250 Section 5.4.1 | At least three spray heads mounted in a water supply rack are used as the test apparatus shown in Figure 5-1. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 55 |  | Spray heads are constructed in accordance with what is shown in Figure 5-2. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 55 |  | DMS is positioned in the focal area of the spray heads. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 55 |  | Water pressure is maintained at 34.5 kPA (5 psi) at each spray head, and water is continuously sprayed for one hour. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| The following criteria are for Luminance Intensity Requirements for DMS. |  |  |  |  |  |
| 56 | 5.4 | Luminous intensity is measured in accordance with the tables in Section 5.4.1 and Section 5.9. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 57 |  | Luminous intensity uniformity is separately met for each pixel color. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 58 |  | DMS luminous intensity appears uniform to the naked eye. |  | If the DMS does not appear uniform, the luminous intensity of the highest and lowest appearing pixels shall be measured and the intensity ratio (11/L2, where L1 > L2) between the two shall be less than 3:1. | Document Review |
|  |  |  |  |  | Init.: |

Document History for:

NEMA TS4 2.2.3-2.2.6, 3.1.1 & 5.4 Checklist

| Rev | Description | Authored and Checked | Reviewed | Approved | Approval Date | Rev More Stringent? |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | New checklist | A. Cramer | W. Geitz D. Bremer | D. Vollmer | 01/15/2025 | N/A |
| 2.0 | Clarified voltage requirements for low and high-voltage tests. Removed parts of 3.1.1 that are to be verified outside of a test report. | A. Cramer | M. DeWitt | M. DeWitt | 11/03/2025 | No |
| 3.0 | Added NEMA type 3R requirements, described in ANSI/NEMA 10250 standard. | A. Cramer | W. Geitz | D. Vollmer | 02/27/2026 | No |